Analysis of thin sections as in Marggraf et al.

Goal: Measure the nearest neighbor distance between the amygdules (or other phenocrysts) and their relative surface density

- Merge microscope pictures

  - Automate/photomerge

  - Flatten image

- Cut thin section to size

  - Draw circle/rectangle using the ellipse tool and adjust to size of thin section

  - Load the circle as a selection

    - Select your shape layer in the Layers panel.

    - Go to Window → Paths to open the Paths panel.

    - You’ll see a Work Path or the shape’s path.

    - Ctrl + Click on the path thumbnail to load it as a selection.

  - Click the underlying layer in the Layers Panel

  - Layer → New → Layer via Cut, cut area will be in a new layer

- Measure area of picture

  - Select the layer: Make sure the layer you want to measure is selected in the Layers panel.

  - Load the selection: Press Ctrl and click the layer's thumbnail to create a selection around its contents.

  - Open the Measurement Log: Go to Window > Measurement Log.

  - Record measurements:

    - Go to Image > Analysis > Record Measurements.

    - A dialog box will appear. Select "Area" (and any other desired data points) and click "OK".

    - The Measurement Log panel will update with the area in pixels.

- Trace amygdules

  - Check layer that should be edited (the picture)

  - Add path in the path panel, trace amygdule with the pen tool

  - Add a new path for each amygdule. Color them with the brush tool (select several ones in the path panel and right click, stroke path)

- Measure surface of amygdules

  - Run via File > Scripts > calculate_path_areas.

  - Check your Desktop for path_areas.txt with all areas in pixels².

  - Used script is in: Windows: C:\\Program Files\\Adobe\\Adobe Photoshop[version]\\Presets\\Scripts

- Calculate surface percentage of amygdules

- Get distance between amygdules (in Photoshop and python)

  - Best: Trace amygdules as paths in a new layer

  - Otherwise: If amygdules are individual paths in the same layer as the picture, proceed in Photoshop and follow:

    - Duplicate layer

    - Select all paths (in the path panel – click all of them and click the dashed circle in the bottom part)

    - Invert selection using Shift + Ctrl + I

    - Delete (removes all image content and keeps the paths/amygdules only)

  - Select all paths (in the paths panel – click all of them and click the dashed circle in the bottom part)

  - Convert each path to a filled shape, select the layer and click Layer → New Fill Layer → Solid Color → White

  - Put solid black as the background: Layer → New Fill Layer → Solid Color black

  - Put the black layer underneath the white path layer

  - Export as .png

  - Run python code

- Resample

  - Image – Image Size – Bicubic Sharper, Unit: pixels, Quantity : 0.5 original